Description of flight level and Wband radar data processing from NOAA P3 aircraft during ATOMIC

Version 4

C. Fairall, S. Pezoa, Ken Moran

2020 July 29

NOAA deployed the P-3 aircraft during the ATOMIC 2020 field program (Jan-Feb). The P-3 dropped dropsondes and AXBT’s, fielded several standard P3 systems, and carried the PSD Wband and WSRA radars.  In this document we will briefly describe some processing that has been done to produce time series summary files from the P-3 flight level sensors and the Wband radar.  An earlier document describes processing of dropsondes and AXBT’s.

Raw data from all P-3 systems are archived at ftp://ftp1.esrl.noaa.gov/psd3/cruises/ATOMIC_2020/P3/

Individual flights are given in directories with the names 2020MMDDI1, with flights on

01 17 20200117I1 17

01 19 20200119I1 19

01 23 20200123I1 23

01 24 20200124I1 24

01 31 20200131I1 31

02 03 20200203I1 34

02 04 20200204I1 35

02 05 20200205I1 36

02 09 20200209I1 40

02 10 20200210I1 41

02 11 20200211I1 42

Within each flight directory are subdirectories:

AVAPS 	Dropsonde raw files

DMT 		Cloud/precipitation microphysics from DMT probes.

FLT LVL	Data from P3 flight level measurements

SFMR		Stepped frequency microwave radiometer

WSRA		Wide Swath Radar Altimeter

AXBT		Raw AXBT files

In this document we describe summary time series files processed from the Wband radar.  The full data sets are archived at ftp://ftp1.esrl.noaa.gov/psd3/cruises/ATOMIC_2020/P3/wband/ .

There is a directory for each flight labeled by day number in 2020.  Each flight directory contains three subdirectories

Mom: Hourly radar moment netcdf files with time series (2 Hz) of profiles (220 30-m range gates) of 1st three Doppler spectral moments.

Motion: netcdf and txt files of the aircraft flight level data for that flight

Raw_images: various png and jpg’s from field processing used mostly to indicate radar performance.  These are preliminary and subject to change.

We have done some preliminary processing of the Wband data mainly to produce time series of bulk cloud properties deduced from radar and IR information.  The IR data are from the P-3 down-looking IR sensor.  The radar processing uses certain variables from the P-3 flight level data.  Because the P-3 files are large and a bit slow to download we have pulled the radar-relevant variables out and written matlab subset for each flight.  These files were produced from the aircraft YYYMMDDI1_AC.nc files that were released a few weeks after the field program.  The matlab mat-files are named ATOMIC_P3_FLTLVLDDD.mat, where DDD is the day number of the flight.  An ‘all’ file with 11 flights combined was also written.  The AC file contains 440 variables but we have saved only 28 in the mat file:

P3_doy=datenum(yy,mm,dd)-datenum(yy-1,12,31);

hors=P3nav.HH; mins=P3nav.MM; secs=P3nav.SS;

P3_hha=hors+mins/60+secs/3600; ii=find(~isnan(P3_hha));

P3_hh=P3_hha(ii);%decimal doy

P3_lat=P3nav.LATref;P3_lat=P3_lat(ii);%latitude deg P3_lon=P3nav.LONref;P3_lon=P3_lon(ii);%longitude deg

P3_altref=P3nav.ALTref;P3_altref=P3_altref(ii);%altitude m

P3_pitchref=P3nav.PITCHref-1.1;P3_pitchref=P3_pitchref(ii); aircraft pitch deg

P3_rollref=P3nav.ROLLref;P3_rollref=P3_rollref(ii);%for left wind up aircraft roll deg

P3_cog=P3nav.TRKd;P3_cog=P3_cog(ii);%COG m/s

P3_Windspeed=P3nav.WSd;P3_Windspeed=P3_Windspeed(ii);%flight level U  m/s

P3_WindDir=P3nav.WDd;P3_WindDir=P3_WindDir(ii);%flight level dir deg from N

P3_sfmrU=P3nav.SfmrWS1;P3_sfmrU=P3_sfmrU(ii);%Microwave 10-m wind 118 m/s

P3_sfmrR=P3nav.SfmrRainRate1;P3_sfmrR=P3_sfmrR(ii);%Rain rate 117 mm/h

P3_sst=P3nav.TRadD1;P3_sst=P3_sst(ii);%119  not sure where this comes from

P3_GS=P3nav.GSd;P3_GS=P3_GS(ii);%SOG in m/s

P3_Hed=P3nav.THDGref;P3_Hed=P3_Hed(ii);%aircraft heading deg from N

P3_TAS=P3nav.TASd;P3_TAS=P3_TAS(ii);%true air speed m/s

P3_press=P3nav.PSc;P3_press=P3_press(ii);%air pressure mb

P3_Td=P3nav.TDc;P3_Td=P3_Td(ii);%dew pt Temp C

P3_Ta=P3nav.TAd;P3_Ta=P3_Ta(ii);%air Temp C

P3_RH=P3nav.HUM_RELd;P3_RH=P3_RH(ii);% rel humidity %

P3_Q=qair3([P3_press P3_Ta P3_RH]);%specific hum computed from rh g/kg

P3_TradD=P3nav.TRadD1;P3_TradD=P3_TradD(ii);%IR sst C

P3_TradS=P3nav.TRadS1;P3_TradS=P3_TradS(ii);%ocean salinity?

P3_cab=P3nav.PCAB1;P3_cab=P3_cab(ii);%P3 cabin pressure mb

P3_wvel=P3nav.UWZd;P3_wvel=P3_wvel(ii);%flt lvl Wind components m/s

P3_uvel=P3nav.UWXd;P3_uvel=P3_uvel(ii);

P3_vvel=P3nav.UWYd;P3_vvel=P3_vvel(ii);

Preliminary analysis shows P-3 and sondes compare well for air temperature and humidity (Figs. 1 and 2).

We have prepared a second time series file from P-3 observations that includes cloud information deduced from Wband radar and IR thermometer.  It also contains basic navigation data for context.  One file is produced for each flight and an ‘all’ file is also produced.  The parameters are listed below:

doymh;%  day of year

hhmh;%decimal hour

P3_latmh;%latitude deg

P3_lonmh;%longitude dew

P3_altrefmh;%m

P3_tasmh;%true airspeed m/s

P3_pitchrefmh;%pitch deg

P3_rollrefmh;%roll deg

P3_pressmh;%pressure mb

P3_sstmh;%raw IR temperature c

nrcsmch;%normalized radar cross section, Corrected for range dependence 94 GHz dB

Us10mh;%sfmr raw 10-m wind speed m/s

Us10mch;%sfmr corrected 10-m wind speed m/s

cloudx2h;%radar-based cloud index, Max SNR+14, positive indicates cloud

cldindxh;%IR-based cloud index=corrected IR-SST negative is cloud

httmh;%radar-based cloud top height km

reftmzh;%wband reflectivity at cloud top, dBZ

wvelmah;%Wband Doppler at cloud top, m/s

tatmh;%Air temperature at cloud top, C, from sounding

tircth;%corrected IR temp at cloud top, C

uatmh;%wind speed at cloud top (m/s) from soundings

tirsst;%estimate of SST computed for IR thermometer, C

rain2;%estimate of rainrate computed from wband nrcsc, mm/hr

mss_smo%smoothed mean square slope computed from nrcs

The first 10 variables are straightforward renditions of P3 data interpolated to the 2 Hz radar time base.  The last 14 variables are derived from a combination of wband, SFMR, IR, and aircraft profile information.

nrcsm is radar cross section computed as per Fairall et al. 2018

where dBZe(0) is the effective reflectivity from the surface return, δR the radar range gate thickness (30 m), 180 a factor converting mm^6/m^3, and 4 is the molecular attenuation at Wband for typical ATOMIC conditions with the aircraft at 3 km altitude.

NRCS is a simple characterization of the scatter of the radar signal from the surface commonly retrieved from satellite or airborne radars.  It depends on the radar wavelength, the incidence angle, and the characteristics of the surface.  Over the ocean that characteristic is the ocean wave spectrum.  The correlation of NRCS with windspeed is the basis of wind speed retrievals from scatterometers.

Cloud2xh is cloud index based on the maximum SNR is the radar column.  The noise level of the radar is nominally -20 dB, but a value of about -15 is needed to ensure a valid cloud return.  By adding 14 to the max SNR we get a number that is positive when a cloud is present below the aircraft.

Cldindxh is computed from the measure IR sensor, P3_ssstmh.  In the absence of clouds, this sensor sees the SST but with and atmospheric correction that depends on the aircraft altitude.  We used the radar cloud index to identify clear situations and did a fit of P3_sstmh to pressure to get a mean atmospheric correction

From this we can estimate the true SST or the IR temperature at cloud top from measurement at any altitude

The pressure at cloudtop is obtained from the radar-derived cloudtop height and the known dependence of pressure on height.  The term Tirc(0)=a is the mean SST.  We also estimate the actual temperature at the radar-derived cloudtop height from the mean air temperature profile, tatm.  If the cloud is sufficiently thick to be black in the IR and fills the full beam sample (about 200 m), then tatm should be the same as tirct.  We also created an IR cloud index

Which is 0 in the absence of clouds.  When clouds are present it is negative because the clouds are colder than SST.  An example from 1 hour of data is shown in Figs.5-6.  Cloudtop height diagnosed from Wband radar return is shown for the same hour in Fig. 7.  In Fig. 8 we show the difference in cloudtop temperature and apparent IR cloudtop temperature vs Wband reflectivity from January 19.  Values of tatm-tirct less than zero indicate the cloud is optically thin or does not completely fill the 0.5 s sample and some radiation from the warm sea surface is adding to the measured IR.

Means square slope is computed from NRCS in rain-free conditions.  We can compute mean square slope from values of nrcs_c restricted to precipitation-free and near-nadir pointing angles:

See the write up P3 Wband and NRCS.

Some caution about this cloud information.  The Wband radar sensitivity is limited and many clouds do not register, particularly when the aircraft is at 7.5 km altitude (about ¼ of the time).  The IR is likely a more sensitive cloud detector in those cases.  When the aircraft is at 3 km, the radar is very sensitive to clouds and likely detects all clouds with dBZ>-35.  Mean cloudtop height and cloud fraction as a function of longitude from P-3 ATOMIC flights are shown in Fig. 9.  The IR cloud fractions is higher than the radar value (grand average 0.18 vs 0.13), but the exact values are sensitive to the cloud-nocloud threshold.  The significance of the longitudinal variations is questionable because we only have a sample of 11 flights and each flight tends to sample a fairly small region.  Also, one or two of the flight days were during cases with, shall we say, somewhat impure classical shallow cumulus conditions near Barabados.  In Fig. 10 we compare the two P3 cloud fraction estimates with lidar ceilometer values from the Ronald H. Brown.  Here we show cloud fraction as a function of 10-m wind speed.  The lidar detects about twice as many clouds as the P3 sensors.  The lidar is optimized for cloud detection and sees clouds about as well as the human eye in bright sunlight. So, it is clear the P3 sensors miss a lot of the smaller clouds.

The file contains corrected values for the SFMR wind speed and the NRCS.  The wind speed has corrected based on comparisons with dropsonde 10-m winds with a simple linear regression.  The NRCS was corrected for saturation of the receiver by the strong surface return when the aircraft was below about 10 km altitude.  The correction depends on aircraft altitude.  Rain rate is derived from Wband attenuation following Fairall et al. 2018.  A smoothed, corrected SFMR wind speed is used to estimate the oceanic normalized cross section, nrcf2.  The attenuation of the radar is the difference in measured and computed nrcs.

u10c_smo=smoothdata(Us10mch,'movmedian',100);

nrcf2=3+9.94-.2*u10c_smo-.004*u10c_smo.^2;

rain2=0.5*(nrcf2-nrcsmch)*1.1/sqrt(1.2)./P3_altrefm*1e3;

The minimum detectable rainrate is about 0.5 mm/hr but the background wanders some so values less than 0.75 mm/hr are not reliable.  Using 0.75 for a threshold this time series give a mean rain occurrence of 1.2% and an overall average rainrate of 0.36 mm/day.  If we use 0.5 mm/hr for the threshold, we get a mean rain occurrence of 2.4% and an overall average rainrate of 0.57 mm/day.

Fairall, C., S. Y. Matrosov, C. R. Williams, E. J. Walsh, 2018: Estimation of rain rate from PSD airborne Doppler W-band radar in CALWATER2.  J. Atmos. Oceanic Tech., 35, 593-608. DOI: 10.1175/JTECH-D-17-0025.1.

Figure  1.  Comparison of dropsonde and P-3 air temperatures for ATOMIC: mean all flights.

Figure 2.  Comparison of dropsonde and P-3 specific humidity for ATOMIC: mean all flights.

Figure 3. Locations of dropsondes for ATOMIC.

Figure 4.  SST values from the ATOMIC AXBT’s on a lat-lon grid.

Figure 5.  Time height cross section from Wband radar on Jan 19 at 1800-1900.  The panels are reflectivity, Doppler velocity, and Doppler width.  The aircraft is flying at 3.1 km.  Clouds are apparent with tops from 1.5 to 3.0 km; yellow streaks going to the surface are rain.

Figure 6.  Time series of cloudx2h (blue) and cldindxmh (red) for the same time as Fig. 5.

Figure 7.  Time series of cloudtop height (httmh) for the same time as Fig. 5.

Figure 8.  Difference in cloudtop temperature and apparent IR cloudtop temperature vs Wband reflectivity from January 19.  Values less than zero indicate the cloud is optically thin or does not completely fill the 0.5 s sample and some radiation from the warm sea surface is adding to the measured IR.

Figure 9.  Mean cloudtop height (upper panel) and cloud fraction (middle panel) as a function of longitude from P-3 ATOMIC flights.  The lower panel is the number of observations.

Figure 10.  Cloud fraction averaged in 10-m wind speed  bins from ATOMIC.  Uplooking lidar ceilometer from Ronald H. Brown (blue); downlooking P3 IR thermometer (red); downlooking Wband radar (yellow).