Stratus 2018 Flux System Ship-Based

The ESRL/PSD Turbulent Flux System

The PSD Flux Measurement System consists of six components (Table 1):

- Solar and infrared radiation sensors. Radiometers are mounted on top of bridge house 16m from Sea Surface.

- Bulk Meteorology sensors (air temperature, relative humidity, atmospheric pressure and precipitation). Instruments are mounted on the top of a portable weather tower 15.39m from sea level.

- One differential GPS units measuring heading and pitch information, installed above pilot house-port side about 15m from sea level.

- A sea surface temperature measurement made with a floating thermistor deployed of port on the port side with outrigger.

| Table 1. Instruments and measurements for air-sea interaction studies |  |  |
| --- | --- | --- |
| Item | System | Measurement |
| 1 | Bulk Meterrology | Air Temp, Relative Humidity,Wind Speed,Wind Direction, Atmospheric Pressure, Rain Rate. |
| 2 | Pyranometer &Pyrgeometer | Downward solar radiative, IR flux |
| 3 | GPS and Heding | Ship SOG, COG,Lat,Lon GPS and Heading |
| 4 | Sea Snake | Near Surface Sea Temp |
| 5 |  |  |
| 6 |  |  |

Instrument measurements are logged in a ship’s lab or in a sea container supplied by ESRL.  The systems will run continuously through the cruise, special attention on Day 100 to 102 of 2018 for Strauts18 data comparison. The ship’s SCS data provided by end og the day files The best situation for obtaining flux data is with the ship going slow ahead and the wind within 45 degrees of the bow.

Figure A-1. ESRL/PSD mast mounted on the Cabo de Hornos, showing the location of weather tower. Sea snake” is on the port side about 15m from the tower.

Figure A-2. ESRL/PSD radiometers mounted on deck above the pilot house about 16.4m from sea level.

Stratus 2018 Flux System Ship-Based Sensors Coefficient Table

| Sensor | Calibration coefficient | Make / Model | Serial Number | Date of calibration |
| --- | --- | --- | --- | --- |
| Precision Spectral Pyranometer | 0.009 | K&Z/ CM22 | 15713050122 | June, 2017 |
| Precision Spectral Pyranometer | 0.00951 | K&Z/CMP22 | 15521170518 | May, 2017 |
| Precision Infrared Radiometer | 0.00392 | Eppley / PIR 1 | 38521F3 | April, 2017 |
| Precision Infrared Radiometer | 0.0026 | Eppley / PIR 2 | 38519F3 | April, 2017 |
| Vaisala Weather Transmitter |  | WXT-520 | L4720496 | Feb, 2017 |
| Temp / Humidity | n/a | Vaisala/HMT335 | C1110008 |  |
| Class A Barometer | n/a | Vaisala/ PTB220 | A2710002 |  |
| Sea Snake thermistor 0C to 40C | C4=0.001399937 C5=0.00237854 C6=0.000000097 | YSI 46031 series | n/a |  |
| GPS and Heading |  | Hemisphere/Crecent VS110 | 072235040014 |  |
| Datalogger |  | Campbell/CR1000 | 7908 |  |

Stratus 2018 Flux measurements result

￼￼